FACULTY OF ENGINEERING

AINSHAMS UNIVERSITY

Major_task

Machine vision

| NAME | ID | contribution |
| --- | --- | --- |
| Mohamed Emad | 18P182 | 33.33% |
| Eslam Elsokary | 18P6123 | 33.33% |
| Zeyad mihamed abdellatif | 20P6963 | 33.33% |

# Table of Contents

Table of Contents	2

Abstract	5

1. Introduction	5

1.1 Project Overview	5

1.2 Problem Statement	5

1.3 Project Objectives	5

2. Dataset	6

2.1 Dataset Description	6

2.2 Dataset Organization	6

2.3 Dataset Split	6

2.4 Data Augmentation	7

3. Milestone 1 — Classical Image Processing	7

3.1 System Architecture	7

3.1 System Architecture	7

4. Image Enhancement	8

Parameters used	9

5. Thresholding	9

5.1 Global Thresholding	9

Important	10

5.2 Otsu Thresholding	10

6. Edge Detection	10

Canny Edge Detector	11

7. Piece Segmentation and Contour Extraction	11

7.1 Foreground Mask	11

7.2 Connected Components	11

7.3 Boundary Tracing	11

7.4 Piece Extraction and Storage	12

8. Piece-Edge Description	12

8.1 Corner Detection	12

8.2 Side Division	12

8.3 Edge Classification	13

8.4 Photometric Signature	14

Parameters used	14

9. Piece-Edge Matching	15

9.1 Shape Compatibility	15

9.2 Colour Compatibility	15

9.3 Combined Compatibility Formula	16

10. Assembly Algorithm	17

10.1 Greedy Best-First Search	17

10.2 Initial Piece Selection	17

10.3 Orientation Resolution	17

10.4 Placement Cost	17

10.5 Tie-Breaking Rule	17

10.6 Dead Ends and Unplaceable Pieces	17

10.7 Best Arrangement Guarantee	17

11. Milestone 1 Results	18

11.1 Image Enhancement Results	18

11.2 Segmentation Results	18

11.3 Matching Results	18

11.4 Reconstruction Results	18

11.5 Reconstruction Quality	18

12. Milestone 2 — Machine Learning	18

13. Dataset Preparation for Machine Learning	18

13.1 Positive Side-Pair Samples	18

13.2 Negative Side-Pair Samples	18

13.3 Input Representation	18

13.4 Augmentation	19

14. Siamese Convolutional Neural Network	19

14.1 Architecture	19

14.2 Input and Output Representation	19

14.3 Loss Function	19

14.4 Training Configuration	19

14.5 Training and Validation Results	19

14.6 Final Model Selection	19

15. Graph Neural Network	20

15.1 Graph Representation	20

15.2 Architecture	20

15.3 Input and Output Representation	20

15.4 Loss Function	20

15.5 Training Configuration	20

15.6 Training and Validation Results	20

15.7 Final Model Selection	20

16. ML-Based Puzzle Assembly	20

16.1 Common Assembly Algorithm	20

16.2 Siamese CNN Assembly	21

16.3 GNN Assembly	21

16.4 Border and Piece-Usage Constraints	21

17. Performance Evaluation	21

17.1 Evaluation Protocol	21

17.2 Neighbour Accuracy	21

17.3 Position Accuracy	21

17.4 Orientation Accuracy	21

17.5 Complete Reconstruction Accuracy	21

17.6 Reconstruction-Quality Score	21

17.7 Execution and Training Time	21

17.8 Model Size and Memory	21

18. Final Comparison	22

18.1 Accuracy Comparison	22

18.2 Computational Efficiency	22

18.3 Effect of Puzzle Size	22

19. Discussion	22

19.1 Classical Method	22

19.2 Siamese CNN	22

19.3 Graph Neural Network	22

19.4 Error Analysis	22

20. Conclusion	23

21. Future Work	23

22. References	23

23. Appendix	23

Appendix A — Important Algorithms / Pseudocode	23

Appendix B — Additional Results	23

Appendix C — Parameter Tables	23

# Abstract

This project focuses on the development of an automated jigsaw puzzle reconstruction system capable of reassembling a scrambled puzzle from its individual pieces. The main challenge is to determine the correct position and orientation of each piece based on its geometric shape and the visual information contained along its edges. The project is divided into two milestones. In Milestone 1, a classical image-processing approach is developed from scratch. The method includes image enhancement, thresholding, edge detection, piece segmentation, contour extraction, edge classification, colour-based edge description, and a matching algorithm that is used to assemble the puzzle. The system is designed to handle pieces presented in different orientations.

In Milestone 2, two machine-learning approaches are developed and evaluated: a Siamese Convolutional Neural Network (CNN) and a Graph Neural Network (GNN). Both models learn compatibility between puzzle-piece edges and provide compatibility scores that are used by the same assembly algorithm to reconstruct the puzzle.

The methods are evaluated using the provided puzzle dataset based on neighbour accuracy, piece position accuracy, orientation accuracy, complete reconstruction accuracy, reconstruction-quality score, execution time, training time, and memory requirements. The final results will be used to determine which approach provides the most accurate and computationally efficient puzzle reconstruction.

# 1. Introduction

## 1.1 Project Overview

The objective of this project is to develop an automated jigsaw puzzle reconstruction system capable of receiving a scrambled set of puzzle pieces and recovering their original arrangement. The project investigates two fundamentally different approaches: classical image-processing techniques and machine-learning techniques.

## 1.2 Problem Statement

The input consists of puzzle pieces that may be shuffled and, for the relevant difficulty tier, presented in different orientations. The system must determine the identity, position, and orientation of every piece and generate the reconstructed puzzle image together with a numerical measure of reconstruction quality.

## 1.3 Project Objectives

The main objectives are to build a reusable classical image-processing library, develop a Siamese Convolutional Neural Network and a Graph Neural Network, use their compatibility predictions within a common assembly algorithm, and compare the three approaches using consistent evaluation criteria.

# 2. Dataset

## 2.1 Dataset Description

Dataset is a set of puzzle that needs to be re arranged

## 2.2 Dataset Organization

The dataset is organized into three main subsets: training, validation, and testing. The training subset is used to develop and tune the reconstruction methods, while the validation subset is used to evaluate intermediate performance and select suitable parameters or model configurations. The testing subset is reserved for the final evaluation and contains data that is not used during model development.

Each sample consists of a scrambled jigsaw puzzle image containing multiple individual puzzle pieces. The objective of the reconstruction system is to identify the individual pieces, determine their relative relationships, resolve their orientations when required, and recover the original arrangement.

For the classical image-processing pipeline, each scrambled puzzle image is processed individually. The input image is converted into RGB and grayscale representations, after which the pieces are segmented from the background and extracted as individual components. Each extracted piece is represented by its mask, contour, normalized orientation, and edge descriptions.

For Milestone 2, the puzzle data is additionally used to generate positive and negative side-pair samples for machine-learning training. Positive samples represent pairs of sides belonging to true neighbouring pieces, while negative samples represent sides that are not neighbours. The training, validation, and testing subsets are maintained consistently for both the Siamese CNN and Graph Neural Network to provide a fair comparison.

The testing subset is used only for the final performance evaluation of the three approaches. The evaluation considers neighbour accuracy, piece position accuracy, orientation accuracy, complete reconstruction accuracy, reconstruction quality, execution time, and computational requirements.

## 2.3 Dataset Split

The same training, validation, and testing split must be used by the Siamese CNN and GNN to ensure a fair comparison.

| Split | Number of Puzzles | Percentage |
| --- | --- | --- |
| Training | 30680 | 83% |
| Validation | 3855 | 10.4% |
| Testing | 725 | 6.6% |

## 2.4 Data Augmentation

Data augmentation was applied only to the training subset in order to increase the diversity of the training samples and improve model generalization. The validation and testing subsets were kept unaugmented so that their performance represents the models' behaviour on genuine unseen data.

For each training image, the original sample set is supplemented with two additional augmented passes. The augmentation process applies random geometric and photometric transformations to the puzzle pieces. Geometric augmentation is performed through random rotation, with the rotation angle sampled from the range −25° to +25°. Photometric augmentation is also applied, including variations in noise, illumination, contrast, and colour. These transformations simulate differences that can occur when puzzle pieces are photographed under different orientations and lighting conditions.

After augmentation, the pieces are processed using the same feature-extraction and labelling procedure as the original images. The side features consist of a shape profile containing 32 samples concatenated with a 32-sample RGB colour strip, producing a common representation for both the Siamese CNN and GNN.

Importantly, augmentation is applied only to the training split. The validation and test sets remain unchanged, preventing augmented versions of the training data from influencing the final evaluation. Both machine-learning models therefore use equivalent training, validation, and testing data conditions, allowing a fair comparison between the Siamese CNN and GNN.

# 3. Milestone 1 — Classical Image Processing

## 3.1 System Architecture

### 3.1 System Architecture

The classical Milestone 1 system is implemented as an end-to-end image-processing pipeline. The reconstruction process begins by loading the scrambled puzzle image and generating both RGB and grayscale representations. Gaussian smoothing is then applied to the grayscale image to reduce noise.

The RGB image is used to generate a foreground mask separating the puzzle pieces from the background. Morphological opening is subsequently applied to remove small unwanted structures. The resulting binary image is processed using an 8-connected-component labelling algorithm. Component statistics are calculated and used to identify and remove unsuitable regions.

Because multiple puzzle pieces may occasionally touch each other in the input image, the implementation includes a dedicated procedure for splitting touching components. Additional filtering is then performed using minimum component area, maximum aspect ratio, and texture-density constraints.

The remaining components are extracted as individual puzzle pieces. Each piece undergoes additional morphological closing and hole filling to improve its mask. Its contour is then traced again and its normalized orientation is calculated. The resulting piece representations are passed to the piece-description stage, where edge information is extracted and used by the assembly algorithm.

Finally, the assembly module determines the arrangement of the pieces, after which the reconstruction module generates the reconstructed image. An intrinsic reconstruction-quality score is then calculated based on the resulting assembly.

The complete processing chain can therefore be summarized as:

Scrambled Image → Gaussian Smoothing → Foreground Segmentation → Morphological Processing → Connected Components → Component Filtering → Piece Extraction → Mask Refinement → Contour Extraction → Orientation Normalization → Edge Description → Edge Matching → Assembly → Reconstruction → Quality Evaluation.

# 4. Image Enhancement

Image enhancement was implemented from scratch using NumPy to improve the quality of the puzzle images before segmentation and feature extraction. The enhancement library provides several independently testable operations: Gaussian filtering, median filtering, histogram equalization, contrast stretching, Laplacian sharpening, and unsharp masking. A general 2D convolution function was also implemented and reused by the filtering and sharpening operations.

For noise reduction, a Gaussian filter was implemented by generating the kernel from its size and standard deviation. The Gaussian kernel is defined as:

G(x,y)=2πσ21​e−2σ2x2+y2​

The kernel is normalized so that its elements sum to one. A 5 × 5 kernel with σ=1.0 is used in the main reconstruction pipeline. Reflective padding is used at the image boundaries.

A 3 × 3 median filter was also implemented to remove impulse noise. Since the median operation is nonlinear and cannot be represented as a conventional convolution, neighbourhood windows are generated and their median values are calculated using NumPy vectorized operations.

The library also calculates the image histogram directly and uses it for histogram equalization. In addition, contrast stretching maps the 2nd and 98th intensity percentiles to the range 0–255, reducing the influence of extreme outliers.

For sharpening, both Laplacian sharpening and unsharp masking were implemented. The Laplacian method uses a 3×3 kernel, while unsharp masking uses a Gaussian blur with a 5×5 kernel, σ=1.0, and sharpening amount of 1.5.

Although all of these operations are available as independent functions for testing, the main reconstruction pipeline uses Gaussian smoothing with a 5 × 5 kernel and σ=1.0 before the segmentation stage.

### Parameters used

| Enhancement operation | Parameters |
| --- | --- |
| Gaussian filter | 5 × 5, σ = 1.0 |
| Median filter | 3 × 3 |
| Histogram | 256 bins |
| Histogram equalization | CDF-based |
| Contrast stretching | 2nd–98th percentile |
| Laplacian sharpening | Amount = 1.0 |
| Unsharp masking | 5 × 5, σ = 1.0, amount = 1.5 |

# 5. Thresholding

## 5.1 Global Thresholding

Global thresholding was implemented to separate the puzzle pieces (foreground) from the background. The method uses a single intensity threshold T for the entire image. Each pixel is classified according to:

B(x,y)={1,0,​I(x,y)>TI(x,y)≤T​

where I(x,y) represents the grayscale intensity of the pixel and T is the selected global threshold.

The resulting binary image provides a foreground mask that can subsequently be processed using morphological operations and connected-component labelling to isolate the individual puzzle pieces.

A global threshold is computationally simple and works well when the foreground and background have sufficiently different intensity distributions. However, because the same threshold is applied to the entire image, its performance can decrease when the image contains non-uniform illumination or significant variations in background brightness.

In the implemented puzzle-reconstruction pipeline, thresholding is followed by morphological opening and connected-component analysis to remove small unwanted regions and identify the individual puzzle pieces.

### Important

## 5.2 Otsu Thresholding

Otsu's method is used to automatically determine a global threshold for separating the puzzle pieces from the background. Instead of selecting the threshold manually, the method analyzes the grayscale intensity histogram and selects the threshold that provides the best separation between the foreground and background classes.

The grayscale image is converted into a binary mask according to the selected threshold:

- Pixel ≤ threshold → background (0)

- Pixel > threshold → foreground (255)

This produces a binary representation of the puzzle scene that can be used by subsequent segmentation stages, such as connected-component labeling and piece extraction.

Otsu thresholding was also verified using unit tests. The tests confirmed that a bright foreground object on a dark, noisy background is correctly detected, while the background is rejected. The implementation also checks that the resulting mask contains only binary values (0 and 255).

# 6. Edge Detection

## Canny Edge Detector

A complete Canny detector is implemented using Gaussian smoothing, gradient computation, non-maximum suppression, double thresholding, and hysteresis-based edge linking.

Edge detection is used in the segmentation stage to identify high-frequency structures and distinguish valid puzzle pieces from unwanted objects or noisy regions. The implemented pipeline uses Canny edge detection as part of the texture-density filtering process.

After connected components are extracted, the edge density within each component is evaluated. Components containing excessively high edge density are considered likely to be clutter or highly textured objects and are removed. This helps retain the relatively smooth regions corresponding to puzzle pieces while rejecting unwanted components.

The edge-density threshold used in the main pipeline is:

Maximum edge density = 0.13

This filtering step is particularly useful because puzzle pieces may contain printed patterns, while unwanted objects can produce significantly higher-frequency edge structures.

# 7. Piece Segmentation and Contour Extraction

## 7.1 Foreground Mask

The foreground mask is generated to separate the puzzle pieces from the background. The image is first processed to identify pixels belonging to the puzzle pieces, while background pixels are suppressed. Otsu thresholding is used to automatically determine a suitable threshold based on the image intensity histogram. The resulting binary mask represents the detected foreground regions and is subsequently used for connected-component labelling.

## 7.2 Connected Components

A from-scratch connected-component labelling algorithm is used to identify individual puzzle pieces in the binary foreground mask. The algorithm scans the image pixel by pixel and assigns a unique label to each connected foreground region. Neighbouring foreground pixels are grouped into the same component, while disconnected regions receive different labels. Each resulting label therefore represents one detected puzzle piece.

## 7.3 Boundary Tracing

After identifying the individual connected components, the boundary of each piece is recovered using a boundary-tracing algorithm. Starting from a boundary pixel of a component, neighbouring pixels are examined sequentially to follow the outer boundary of the piece. The process continues until the starting point is reached again, producing an ordered sequence of contour points representing the shape of the puzzle piece.

## 7.4 Piece Extraction and Storage

Each detected puzzle piece is extracted using its bounding box and stored together with the information required for subsequent processing and matching.

| Stored Attribute | Description |
| --- | --- |
| Piece Image | Cropped image containing the detected puzzle piece within its bounding box. |
| Piece Mask | Binary mask indicating the foreground pixels belonging to the piece. |
| Contour | Ordered set of boundary points representing the outer shape of the piece |
| Bounding Box | Minimum rectangular region enclosing the detected piece. |
| Normalized Orientation | Orientation-adjusted representation used to provide a consistent reference for comparing and matching pieces. |

# 8. Piece-Edge Description

## 8.1 Corner Detection

The four corners of each puzzle piece are detected using the minimum-area bounding rectangle of the piece's convex hull. The ordered boundary contour obtained through Moore-neighbour tracing is first converted from image coordinates (row, column) to Cartesian coordinates (x, y). An Andrew monotone-chain convex hull is then calculated from the contour points.

The minimum-area bounding rectangle is obtained by testing the orientation of each convex-hull edge. The orientation that produces the smallest bounding-rectangle area is selected. The four corners of this rectangle are then calculated from its centre, width, height, and rotation angle.

Because the bounding rectangle represents the stable approximately square body of the puzzle piece, rather than its smaller tab and blank features, it provides a robust estimate of the four true piece corners. Each calculated rectangle corner is subsequently snapped to the nearest point on the actual traced contour, producing four corner indices that correspond to real contour points.

## 8.2 Side Division

Once the four corner indices have been identified, the ordered contour is divided into four consecutive side segments. Each side consists of the contour points between two consecutive corners, including both endpoints.

The sides follow the original clockwise ordering of the Moore-neighbour contour. The final side wraps around from the fourth corner back to the first corner.

For subsequent comparison, each side can be resampled according to its arc length. The implementation uses 32 evenly spaced samples when calculating the geometric shape profile and colour signature.

## 8.3 Edge Classification

First, a straight reference line is defined between the two endpoints of the side. The perpendicular signed distance of every contour point from this line is calculated. The normal direction is chosen using the piece centroid so that positive distance represents a deviation outward from the piece, while negative distance represents a deviation toward the piece interior.

The maximum outward and inward deviations are calculated:

dout​=max(di​) din​=max(−di​)

The flatness threshold is defined relative to the side's straight-line length:

Tflat​=0.1L

where L is the distance between the two corner points.

The classification is then:

- Flat: both outward and inward deviations are below 0.1L.

- Tab: the outward deviation is at least 0.1L and is greater than or equal to the inward deviation.

- Blank: the inward deviation is at least 0.1L and is greater than the outward deviation.

Thus, the classification is scale-relative rather than based on a fixed number of pixels.

| Edge Type | Geometric Criterion | Threshold/Parameter |
| --- | --- | --- |
| Tab | Outward deviation dominates and exceeds the flatness threshold. | dout​≥din​, dout​≥0.1L |
| Blank | Inward deviation dominates and exceeds the flatness threshold. | din​>dout​, din​≥0.1L |
| Flat | Neither outward nor inward deviation reaches the threshold. | dout​<0.1L and din​<0.1L |

## 8.4 Photometric Signature

A photometric signature is extracted from the interior of each side to capture the printed colour pattern near the puzzle boundary.

Each side contour is first resampled to 32 equally spaced points according to arc length. For every sample, a point is then selected 4 pixels inward from the contour, in the direction of the piece centroid. This prevents the sampled colour from coming from the background surrounding the piece.

The resulting 32 colour samples form a fixed-length colour strip. For colour images, the samples are stored directly in RGB colour space as 8-bit values.

Therefore, the photometric signature has the form:

C={C1​,C2​,…,C32​}

where each Ci​ is an RGB colour vector.

During matching, one strip is reversed because the two sides of a physical puzzle connection are traversed in opposite directions.

[FIGURE PLACEHOLDER: Colour-strip sampling illustration]

### Parameters used

| Parameter | Value |
| --- | --- |
| Sampling method | Arc-length resampling |
| Number of samples | 32 |
| Inward sampling offset | 4 pixels |
| Colour space | RGB |
| Signature size | 32 RGB samples |
| Orientation during matching | One side reversed |

# 9. Piece-Edge Matching

## 9.1 Shape Compatibility

Shape compatibility is calculated using the signed geometric profile of each side.

Each side is resampled to N=32 points. For every point, the perpendicular deviation from the straight line connecting the two corners is calculated. A positive value represents an outward tab-like deformation, while a negative value represents an inward blank-like deformation.

For two candidate sides, one profile is reversed because the two sides are traversed in opposite directions when placed together. The profiles are then added because a tab and a blank should have opposite deviations.

The shape error is:

Eshape​=L2N1​∑t=1N​[PA​(t)+PB​(N−1−t)]2​​

where:

- PA​(t) is the signed profile of side A,

- PB​(t) is the signed profile of side B,

- N=32,

- L=(LA​+LB​)/2 is the average straight-line length of the two sides.

A perfect geometric complement produces:

PA​(t)=−PB​(N−1−t)

and therefore:

Eshape​=0

Flat sides are not considered possible neighbouring sides.

## 9.2 Colour Compatibility

Colour compatibility measures how smoothly the printed pattern continues across the proposed connection.

The two 32-sample RGB strips are compared after reversing the second strip:

Ecolour​=2552N1​∑t=1N​∥CA​(t)−CB​(N−1−t)∥2​​

where:

- CA​(t) is the RGB vector of sample t on side A,

- CB​(N−1−t) is the reversed RGB vector of side B,

- N=32.

The division by 2552 normalizes the RGB squared difference.

A lower colour score indicates greater similarity between the two printed regions.

## 9.3 Combined Compatibility Formula

the final edge compatibility score combines geometric and photometric errors:

Etotal​=0.6Eshape​+0.4Ecolour​​

The selected weights are therefore:

- Shape: 0.6

- Colour: 0.4

The larger shape weight reflects the fact that physical compatibility is more restrictive than colour similarity. Lighting, compression, and printing variations can produce colour differences even for correct neighbours, whereas an incorrect tab/blank geometry is a stronger indication of an invalid connection.

| Component | Formula | Weight |
| --- | --- | --- |
| Shape compatibility | Eshape​=L2321​∑t=132​[PA​(t)+PB​(32−t)]2​ | 0.6 |
| Colour compatibility | Ecolour​=2552321​∑t=132​∥CA​(t)−CB​(32−t)∥2​ | 0.4 |

For implementation purposes, the matching system only considers complementary tab/blank pairs. Tab-tab, blank-blank, and any pair involving a flat edge are rejected and assigned an infinite score.

9.4 Matching Examples

A correct match should exhibit both geometric complementarity and photometric continuity. The tab profile of one side should approximately cancel the reversed blank profile of the neighbouring side, resulting in a low shape error. At the same time, the colour strips should show a similar printed pattern across the proposed connection, producing a low colour error.

An incorrect match will generally produce a larger geometric mismatch, a larger colour mismatch, or both, resulting in a higher combined compatibility score.

# 10. Assembly Algorithm

## 10.1 Greedy Best-First Search

[Describe the candidate-generation and placement process. At each iteration, select the unused piece and orientation that minimize the accumulated edge dissimilarity with already placed neighbours.]

## 10.2 Initial Piece Selection

[Explain how a corner or border piece is selected and how border constraints are recognized.]

## 10.3 Orientation Resolution

For rotated puzzles, each candidate piece is evaluated under the allowed orientations, typically 0°, 90°, 180°, and 270°. State the exact orientation representation used by the implementation.

## 10.4 Placement Cost

[Give the exact cost function used for a candidate placement, including all already-placed neighbours that contribute to the cost.]

## 10.5 Tie-Breaking Rule

[State the exact deterministic tie-breaking rule.]

## 10.6 Dead Ends and Unplaceable Pieces

[Explain what the algorithm does when no candidate satisfies the placement constraints.]

## 10.7 Best Arrangement Guarantee

[Explain how the best complete or partial arrangement found is stored and returned even if assembly terminates without a complete solution.]

[FIGURE PLACEHOLDER: Assembly process / placement grid]

# 11. Milestone 1 Results

## 11.1 Image Enhancement Results

[FIGURE PLACEHOLDER: Representative enhancement results]

## 11.2 Segmentation Results

[FIGURE PLACEHOLDER: Representative segmentation results]

## 11.3 Matching Results

| Metric | Result |
| --- | --- |
| — | — |
| — | — |
| — | — |
| — | — |
| — | — |

## 11.4 Reconstruction Results

[FIGURE PLACEHOLDER: Original / scrambled / reconstructed comparison]

## 11.5 Reconstruction Quality

[Define the numerical reconstruction-quality metric actually implemented. Report the formula, interpretation, and values for representative puzzles.]

| Puzzle | Neighbour Accuracy | Position Accuracy | Orientation Accuracy | Quality Score | Execution Time |
| --- | --- | --- | --- | --- | --- |
| — | — | — | — | — | — |
| — | — | — | — | — | — |
| — | — | — | — | — | — |

# 12. Milestone 2 — Machine Learning

Milestone 2 develops two fundamentally different machine-learning approaches: a Siamese Convolutional Neural Network and a Graph Neural Network. Both models use the same dataset split and their compatibility scores are passed to the same assembly algorithm.

# 13. Dataset Preparation for Machine Learning

## 13.1 Positive Side-Pair Samples

[Explain how true neighbouring sides are identified and converted into positive training samples.]

## 13.2 Negative Side-Pair Samples

[Explain how non-neighbouring side pairs are sampled. State the positive/negative ratio.]

## 13.3 Input Representation

[Describe the exact tensors/features supplied to both models and identify any differences required by their architectures.]

## 13.4 Augmentation

[List the actual augmentation parameters used for the training set.]

| Augmentation | Parameter | Applied To |
| --- | --- | --- |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |

# 14. Siamese Convolutional Neural Network

## 14.1 Architecture

[Describe the two branches, shared weights, embedding size, comparison layer, and final prediction layer.]

[FIGURE PLACEHOLDER: Siamese CNN architecture]

## 14.2 Input and Output Representation

[State input dimensions, channels, normalization, and output definitions: neighbour prediction, compatibility score, matching side, and relative orientation.]

## 14.3 Loss Function

[State the selected loss function and give its mathematical formulation. Explain why it is appropriate for the compatibility task.]

## 14.4 Training Configuration

| Parameter | Value | Justification |
| --- | --- | --- |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |

## 14.5 Training and Validation Results

[FIGURE PLACEHOLDER: Siamese CNN training/validation curves]

| Metric | Training | Validation | Testing |
| --- | --- | --- | --- |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |

## 14.6 Final Model Selection

[Explain the validation criterion used to select the final checkpoint. The unseen test set must not be used for model selection.]

# 15. Graph Neural Network

## 15.1 Graph Representation

[Define graph nodes, candidate relationships/edges, node features, edge features, and labels.]

[FIGURE PLACEHOLDER: GNN graph representation]

## 15.2 Architecture

[Describe the GNN layers/message passing mechanism, aggregation, and prediction head.]

[FIGURE PLACEHOLDER: GNN architecture]

## 15.3 Input and Output Representation

[State the exact input features and outputs, including neighbour prediction, compatibility score, matching side, and relative orientation where applicable.]

## 15.4 Loss Function

[State and justify the selected loss function.]

## 15.5 Training Configuration

| Parameter | Value | Justification |
| --- | --- | --- |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |
| — | — | — |

## 15.6 Training and Validation Results

[FIGURE PLACEHOLDER: GNN training/validation curves]

| Metric | Training | Validation | Testing |
| --- | --- | --- | --- |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |

## 15.7 Final Model Selection

[Explain the validation-based checkpoint/model selection method.]

# 16. ML-Based Puzzle Assembly

## 16.1 Common Assembly Algorithm

To ensure a fair comparison, the Siamese CNN and GNN compatibility scores are passed to the same assembly strategy. The assembly must use every piece once, respect border constraints, and maximize compatibility or minimize matching error.

## 16.2 Siamese CNN Assembly

[Explain how CNN scores are converted into candidate placements and orientations.]

## 16.3 GNN Assembly

[Explain how GNN scores are converted into candidate placements and orientations.]

## 16.4 Border and Piece-Usage Constraints

[Explain how border constraints and the one-use-per-piece constraint are enforced.]

[FIGURE PLACEHOLDER: CNN and GNN reconstruction examples]

# 17. Performance Evaluation

## 17.1 Evaluation Protocol

Final evaluation is performed only on the unseen testing puzzles. The same testing set and evaluation conditions are used for the classical method, Siamese CNN, and GNN wherever applicable.

## 17.2 Neighbour Accuracy

Neighbour accuracy = correct neighbour predictions / total evaluated neighbour predictions. State the exact evaluation protocol used in the experiment.

## 17.3 Position Accuracy

[Define how a piece is considered correctly positioned.]

## 17.4 Orientation Accuracy

[Define how a piece is considered correctly oriented.]

## 17.5 Complete Reconstruction Accuracy

[Define the condition under which an entire puzzle is counted as correctly reconstructed.]

## 17.6 Reconstruction-Quality Score

[State the exact quality metric and normalization.]

## 17.7 Execution and Training Time

| Method | Inference/Execution Time | Training Time |
| --- | --- | --- |
| — | — | — |
| — | — | — |
| — | — | — |

## 17.8 Model Size and Memory

| Method | Parameters | Model Size | Peak/Required Memory |
| --- | --- | --- | --- |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |

# 18. Final Comparison

| Metric | Classical | Siamese CNN | GNN |
| --- | --- | --- | --- |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |
| — | — | — | — |

## 18.1 Accuracy Comparison

[Interpret the numerical results in terms of actual puzzle reconstruction performance.]

## 18.2 Computational Efficiency

[Compare execution time, training cost, model size, and memory requirements.]

## 18.3 Effect of Puzzle Size

[Evaluate how performance changes as the number of pieces increases.]

| Puzzle Size | Classical Quality | CNN Quality | GNN Quality | Classical Time | CNN Time | GNN Time |
| --- | --- | --- | --- | --- | --- | --- |
| — | — | — | — | — | — | — |
| — | — | — | — | — | — | — |
| — | — | — | — | — | — | — |
| — | — | — | — | — | — | — |

[FIGURE PLACEHOLDER: Performance vs puzzle size graph]

# 19. Discussion

## 19.1 Classical Method

[Discuss strengths, limitations, sensitivity to image quality, interpretability, and scalability.]

## 19.2 Siamese CNN

[Discuss learned visual compatibility, robustness, training requirements, and computational cost.]

## 19.3 Graph Neural Network

[Discuss relational/global context, scalability, graph construction cost, and prediction quality.]

## 19.4 Error Analysis

[Show representative failure cases and explain the causes, such as similar colours, ambiguous shapes, segmentation errors, incorrect orientation, or accumulated assembly errors.]

[FIGURE PLACEHOLDER: Representative failure cases]

# 20. Conclusion

[Summarize what was implemented in Milestone 1 and Milestone 2. State the final performance of all three approaches, identify the most accurate method, identify the most computationally efficient method, and explain how performance changes with puzzle size.]

# 21. Future Work

- Improve global assembly optimization beyond greedy placement.

- Use more robust contour/corner representations.

- Investigate learned edge and contour embeddings.

- Increase dataset diversity and training size.

- Improve handling of visually similar neighbouring edges.

- Explore transformer-based or end-to-end puzzle reconstruction approaches.

- Parallelize compatibility computation and assembly.

# 22. References

[Insert IEEE-style references for the image-processing algorithms, Canny/Sobel/Prewitt, Otsu thresholding, Siamese networks, GNNs, selected loss functions, frameworks, and the supplied dataset.]

| Reference No. | Reference |
| --- | --- |
| — | — |
| — | — |
| — | — |
| — | — |
| — | — |

# 23. Appendix

## Appendix A — Important Algorithms / Pseudocode

[Insert pseudocode for enhancement, connected components, contour tracing, edge matching, and assembly.]

## Appendix B — Additional Results

[Insert additional test puzzles, intermediate images, confusion matrices, and reconstruction examples.]

## Appendix C — Parameter Tables

[Insert the complete final parameter configuration used for reproducibility.]